Flasher Board CPLD Functions

August 8, 2003	N. Kitamura

CPLD I/O definition

signal		I/O	reset value		description

addr[5..0]	I				address bus to DOMMB

data[7..0]	I/O				data bus to DOMMB

nWR		I

nRD		I

clock		I				clock from DOMMB (20 MHz)

attn		O	0			request attention to DOMMB

DCDCEN	O	0 (TBR)		DC/DC enable

DCDCON	O	0 (TBR)		DC/DC on

MOSI		O	‘Z’ (TBR)		MAX5438 digital pot

SCLK		O	‘Z’ (TBR)		MAX5438 digital pot

nCS		O	‘Z’ (TBR)		MAX5438 digital pot

OneW		I/O	‘Z’			DS2401

pre_trig	I	0			a.k.a. AUX_RESET

LED_ON	O	pre_trig		a.k.a. LIGHTS_OUT

trig		I	0			trigger from DOMMB

TRIGGER	O	trig			Out to flasher

DP[7..0]	O	0x00 (TBR)		data bus to DS1023 (adjustable delay)

LE_DP	O	1 (TBR)		latch enable for DP

L[11..0]	O	“0…0” (TBR)

MUX0		O	0 (TBR)

MUX1		O	0 (TBR)

ENMUXA	O	0 (TBR)

ENMUXB	O	0 (TBR)

ENMUXC	O	0 (TBR)

ENMUXD	O	0 (TBR)

ENMUXF	O	0 (TBR)

SDMUX	O	0 (TBR)

Registers

| Address | R/W | Description | Bits definition |
| --- | --- | --- | --- |
| addr_status | R | Flasher board status byte | (Undefined) |
| addr_reset | W | Write any value to this address causes a soft-reset |  |
| addr_cpld_id | R | Flasher board firmware version | = “0x01” (TBR) |
| addr_spi | W | Write a value to the SPI device | Alternative implementation of SPI (To be implemented) |
| addr_mode_select | W | Select mode (normal, spi, 1wire) | (bit1, bit0) “xxxxxx00” for mode_normal “xxxxxx01” for mode_spi “xxxxxx10” for mode_spi |
| addr_dcdc_control | W | dc/dc control (onoff, enable) | bit0 = bit_dcdc_en (enable) bit1 = bit_dcdc_on (on/off) |
| addr_delay_adjust | W | DP[0..7] |  |
| addr_delay_config0 | W | L[0..3] |  |
| addr_delay_config1 | W | L[4..7] |  |
| addr_delay_config2 | W | L[8..11] |  |
| addr_mux | W | Flasher board multiplexer enable bits | bit0 = bit_mux_0 (MUX0) bit1 = bit_mux_1 (MUX1) bit2 = bit_mux_a (ENMUXA) bit3 = bit_mux_b (ENMUXB) bit4 = bit_mux_c (ENMUXC) bit5 = bit_mux_d (ENMUXD) bit6 = bit_mux_f (ENMUX_Final) bit7 = bit_mux_sd (SDMUX) |