# EHWD2005 New Drill Head X Data Format

| Number | Item Name | Description | Bytes | Format |
| --- | --- | --- | --- | --- |
|  | Drill ID | Single Byte ID for Drill: P = Primary Amanda Drill (Not Used) S = Secondary Amanda Drill (Not Used) R = EHWD Refurbished Amanda Drill X = EHWD New Drill Head 1 Y = EHWD New Drill Head 2 | 1 | ASCII |
|  | Timer | 25 millisecond tick counter | 10 | ASCII - Decimal |
|  | System Status | G = Good B = Bad | 1 | ASCII |
|  | Power Supply Heat Sink Temperature Status | G = Good B = Bad | 1 | ASCII |
|  | Power Supply Heat Sink Temperature Data | Unscaled Raw ADC output of heat sink temperature sensor signal | 3 | Hex ASCII |
|  | MLDT1 Status | G = Good B = Bad | 1 | ASCII |
|  | MLDT1 Data | Unscaled Raw ADC output of first MLDT sensor signal | 3 | Hex ASCII |
|  | MLDT2 Status | G = Good B = Bad | 1 | ASCII |
|  | MLDT2 Data | Unscaled Raw ADC output of second MLDT sensor signal | 3 | Hex ASCII |
|  | Load Cell Status | G = Good B = Bad | 1 | ASCII |
|  | Load Cell Data | Unscaled Raw ADC output of amplified load cell signal | 3 | Hex ASCII |
|  | Well (Drill Hole) Temperature Status | G = Good B = Bad | 1 | ASCII |
|  | Well (Drill Hole) Temperature Data | Unscaled Raw ADC output of well (drill hole) temperature sensor signal | 3 | Hex ASCII |
|  | Hose Water Temperature Status | G = Good B = Bad | 1 | ASCII |
|  | Hose Water Temperature Data | Unscaled Raw ADC output of hose water temperature sensor signal | 3 | Hex ASCII |
|  | Power Supply Input Voltage Status | G = Good B = Bad | 1 | ASCII |
|  | Power Supply Input Voltage Data | Unscaled Raw ADC output of power supply input voltage signal | 3 | Hex ASCII |
|  |  |  | 40 |  |

| Number | Item Name | Description | Bytes | Format |
| --- | --- | --- | --- | --- |
|  | Main Regulator Output Voltage Status | G = Good B = Bad | 1 | ASCII |
|  | Main Regulator Output Voltage Data | Unscaled Raw ADC output of main regulator voltage signal (nominally 24 VDC) | 3 | Hex ASCII |
|  | Cavity Air Pressure Sensor Status | G = Good B = Bad | 1 | ASCII |
|  | Cavity Air Pressure Sensor Data | Unscaled Raw ADC output of cavity air pressure sensor signal | 3 | Hex ASCII |
|  | TSeal 1 Pressure Sensor Status | G = Good B = Bad | 1 | ASCII |
|  | TSeal 1 Pressure Sensor Data | Unscaled Raw ADC output of TSeal 1 pressure sensor signal | 3 | Hex ASCII |
|  | TSeal 2 Pressure Sensor Status | G = Good B = Bad | 1 | ASCII |
|  | TSeal 2 Pressure Sensor Data | Unscaled Raw ADC output of TSeal 2 pressure sensor signal | 3 | Hex ASCII |
|  | Well (Drill Hole) Pressure Sensor Status | G = Good B = Bad | 1 | ASCII |
|  | Well (Drill Hole) Pressure Sensor Message ID | Well (drill hole) pressure sensor message IDl | 5 | ASCII |
|  | Well (Drill Hole) Pressure Sensor Data | Scaled well (drill hole) pressure sensor signal (PSI) | 11 | Signed Decimal ASCII |
|  | Watson Nav Pack Status | G = Good B = Bad | 1 | ASCII |
|  | Watson Nav Pack Message ID | Watson Nav Pack message IDl | 1 | ASCII |
|  | Watson Nav Pack: Bank Tilt Reading | Scaled Watson Nav Pack Bank Tilt Reading (Degrees) | 6 | Signed Decimal ASCII with leading space |
|  | Watson Nav Pack: Elevation Tilt Reading | Scaled Watson Nav Pack Elevation Tilt Reading (Degrees) | 6 | Signed Decimal ASCII with leading space |
|  | Watson Nav Pack: Heading Reading | Scaled Watson Nav Pack Heading Reading (Degrees) | 6 | Unsigned Decimal ASCII with leading space |
|  | Watson Nav Pack: X Axis Magnetic Field Reading | Scaled Watson Nav Pack X Axis Magnetic Field Reading (milligauss) | 7 | Signed Decimal ASCII with leading space |
|  | Watson Nav Pack: Y Axis Magnetic Field Reading | Scaled Watson Nav Pack Y Axis Magnetic Field Reading (milligauss) | 7 | Signed Decimal ASCII with leading space |
|  | Watson Nav Pack: Z Axis Magnetic Field Reading | Scaled Watson Nav Pack Z Axis Magnetic Field Reading (milligauss) | 7 | Signed Decimal ASCII with leading space |
|  | Watson Nav Pack: Temperature Reading | Scaled Watson Nav Pack Temperature Reading (degrees C) | 6 | Signed Decimal ASCII with leading space |
|  | End of Message ID and Checksum |  | 3 | ASCII |