Physical Sciences Laboratory

3725 Schneider Drive

Stoughton,  WI   53859-3908

608-877-2200

http://www.psl.wisc.edu

From:		Jack Ambuel

Phone:		608-877-2271

Fax:		608-877-2201

Email: 		jambuel@psl.wisc.edu

To:		Chuck Rentmeesters; Andrew Wagner, Mark Thoma; Bob Morse; Anupam Singh;

Ted Shultz, Phil Robl

Attached are the calibration equations and coefficients for the 2005-2006 drilling season for the EWHD Refurbished Drill Head.

Jack Ambuel

Electrical Engineer

9/29/2005

Variable: Heat Sink Temperature

Calibration Equation and Coefficients:

HeatSinkTemp = HeatSinkTempSlope∗HeatSinkValue + HeatSinkTempIntercept                           where     HeatSinkTemp               = Temperature in Degrees F                                         HeatSinkValue                = Raw Reading from ADC                                           HeatSinkTempSlope      =+0.076434508                                          HeatSinkTempIntercept =+13.8627957

Variable: Hose Water Temperature

Calibration Equation and Coefficients:

HoseWaterTemp = HoseWaterTempSlope∗HoseWaterValue + HoseWaterTempIntercept                           where     HoseWaterTemp                = Temperature in Degrees F                                         HoseWaterValue                 = Raw Reading from ADC                                           HoseWaterTempSlope       =+0.076434508                                          HoseWaterTempIntercept  =+13.8627957

Variable: Drill Hole Water Temperature

Calibration Equation and Coefficients:

HoleWaterTemp = HoleWaterTempSlope∗HoleWaterValue + HoleWaterTempIntercept                           where     HoleWaterTemp                = Temperature in Degrees F                                         HoleWaterValue                 = Raw Reading from ADC                                           HoleWaterTempSlope       =+0.076434508                                          HoleWaterTempIntercept  =+13.8627957

Variable: Input Voltage

Calibration Equation and Coefficients:

InputVoltage = InputVoltageSlope∗InputVoltageValue + InputVoltageIntercept                           where     InputVoltage                    = Voltage in Volts                                         InputVoltageValue           = Raw Reading from ADC                                           InputVoltageSlope           =+0.017700195                                          InputVoltageIntercept      =0

Variable: Output Voltage

Calibration Equation and Coefficients:

OutputVoltage = OutputVoltageSlope∗OutputVoltageValue + OutputVoltageIntercept                           where     OutputVoltage                    = Voltage in Volts                                         OutputVoltageValue           = Raw Reading from ADC                                           OutputVoltageSlope           =+0.007324219                                          OutputVoltageIntercept      =0

Variable: Cavity Air Pressure

Calibration Equation and Coefficients:

CavityAirPress = CavityAirPressSlope∗CavityAirPressValue + CavityAirPressIntercept                           where     CavityAirPress                    = Pressure in PSI                                         CavityAirPressValue           = Raw Reading from ADC                                           CavityAirPressSlope           =−0.007781982                                          CavityAirPressIntercept      =+30

Variable: Load Cell

Calibration Equation and Coefficients:

Upper Range: 0 ≤   LoadCellValue   <  3000   (Corresponding to  >  600 pounds)LoadCell = LoadCellUpperSlope∗LoadCellValue + LoadCellUpperIntercept                           where     LoadCell                               = Tension in Pounds                                         LoadCellValue                      = Raw Reading from ADC                                           LoadCellUpperSlope           =−0.858497489                                          LoadCellUpperIntercept      =+3181LowerRange: LoadCellValue   ≥  3000    (Corresponding to  ≤  600 pounds)LoadCell = LoadCellLowerSlope∗LoadCellValue + LoadCellLowerIntercept                           where     LoadCell                               = Tension in Pounds                                         LoadCellValue                      = Raw Reading from ADC                                           LoadCellLowerSlope           =−0.869616288                                          LoadCellLowerIntercept      =+3205

Variable: TSeal1 Pressure Sensor

Calibration Equation and Coefficients:

Upper Range:  TSeal1Value  ≥  60   (Corresponding to  ≥  160 PSI)TSeal1 = TSeal1UpperSlope∗TSeal1Value + TSeal1UpperIntercept                           where     TSeal1                               = Pressure  in PSI                                         TSeal1Value                      = Raw Reading from ADC                                           TSeal1UpperSlope           =+3.411587865                                          TSeal1UpperIntercept      =−41.631188934LowerRange: TSeal1Value   <  60    (Corresponding to  <  160 PSI)TSeal1 = TSeal1LowerSlope∗TSeal1Value + TSeal1LowerIntercept                           where    TSeal1                               = Pressure  in PSI                                         TSeal1Value                      = Raw Reading from ADC                                           TSeal1LowerSlope           =+3.552631579                                          TSeal1LowerIntercept      =−52.5

Variable: TSeal2 Pressure Sensor

Calibration Equation and Coefficients:

Upper Range:  TSeal2Value  ≥  60   (Corresponding to  ≥  410 PSI)TSeal2 = TSeal2UpperSlope∗TSeal2Value + TSeal2UpperIntercept                           where     TSeal2                               = Pressure  in PSI                                         TSeal2Value                      = Raw Reading from ADC                                           TSeal2UpperSlope           =+3.342573485                                          TSeal2UpperIntercept      =+55.697414899LowerRange: TSeal2Value   <  60    (Corresponding to  <  410 PSI)TSeal2 = TSeal2LowerSlope∗TSeal2Value + TSeal2LowerIntercept                           where    TSeal2                               = Pressure  in PSI                                         TSeal2Value                      = Raw Reading from ADC                                           TSeal2LowerSlope           =+9.642857143                                          TSeal2LowerIntercept      =−168.214285714

Variable: MLDT1 – Hole Diameter Sensor

Calibration Equation and Coefficients:

Upper Range:  MLDT1Value  ≥  1916   (Corresponding to  ≥  25 inches)MLDT1 = MLDT1UpperSlope∗MLDT1Value + MLDT1UpperIntercept                           where     MLDT1                             = Diameter in Inches                                         MLDT1Value                    = Raw Reading from ADC                                          MLDT1UpperSlope           =+0.005424733475480                                         MLDT1UpperIntercept      =+14.6150471215352LowerRange:  MLDT1Value  <  1916   (Corresponding to  <  25 inches)MLDT1 = U0 lnU1∗ MLDT1Value+ U2                           where    MLDT1                               = Diameter in Inches                                         MLDT1Value                     = Raw Reading from ADC                                           U0                                      =+11.5                                          U1                                      =+0.00285                                          U2                                      =+5.5

Variable: HoseWaterPressure – Hose Water Pressure Sensor

Calibration Equation and Coefficients:

Upper Range:  HWPValue  >  196   (Corresponding to  >  252 PSI)   where HWP = Hose Water Pressure HWP = HWPUpperSlope∗HWPValue + HWPUpperIntercept                           where     HWP                               = Pressure  in PSI                                         HWPValue                      = Raw Reading from ADC                                          HWPUpperSlope             =+1.229439598                                         HWPUpperIntercept        =+17LowerRange: HWPValue    ≤  60    (Corresponding to  ≤  252 PSI)HWP = HWPLowerSlope∗HWPValue + HWPLowerIntercept                           where      HWP                               = Pressure  in PSI                                         HWPValue                      = Raw Reading from ADC                                          HWPLowerSlope             =+1.688888889                                         HWPLowerIntercept        =−78

Variable: Sensotek Accelerometer (Accelerometer #1) Readings (Accel1 through Accel5)

Calibration Equation and Coefficients:

Acceleration = SensotekSlope∗SensotekValue + SensotekIntercept                           where      Acceleration                     = Acceleration in g                                            SensotekValue                  = Raw Reading from ADC                                           SensotekSlope                  =+0.001236197                                          SensotekIntercept             =−2.97

Variable: Kistler Accelerometer (Accelerometer #2) Reading (HiLevel3 Amp)

Calibration Equation and Coefficients:

Acceleration = KistlerSlope∗KistlerValue + KistlerIntercept                           where      Acceleration                  = Acceleration in g                                            KistlerValue                  = Raw Reading from ADC                                           KistlerSlope                  =+0.001234374                                          KistlerIntercept             =−3.02